BASE

SHOULDER/WAIST

ELBOW1

ELBOW1

GRIPPER

3D MODELLING OF 5DOF ROBOTIC ARM

BASE OF THE ROBOTIC ARM

- Draw two circles, one with a diameter of 90mm and the other with a diameter of 121.25mm, and then proceed to the "features" section and extrude both circles to a thickness of 4mm.

- Draw  a circle with a diameter of 90mm and another circle with a diameter of 98mm. Then, select features and extrude on the second circle to a height of 52mm.

- Create a reference plane : Go to reference geometry  select plane Select face and distance of the plane 20mmselect flip offset

Sketch on reference plane : go to front plane   sketch draw a line  draw the servo motor braces as shown in the fig

draw support for servo motor braces and extrude it to 10mm

create screw slots on the servo braces design screw space with following dimensions and extrude cut to 5mm.

SHOULDER OF THE ROBOTIC ARM

- Sketch a circle of 97mm  extrude to 4mm -> Select the plane and draw rectangle of 22*60mm

- Draw the reference line on rectangle with the height of 50 mm -> sketch-> spline-> draw the shoulder shape as shown in the fig -> features -> extrude boss base -> extrude to 5mm

- Create a plane reference of 45degree for the arm : draw a line on shoulder plane and by using smart dimension change to  45 degree

- go to reference geometry planeselect first reference(line sketch) as line at 45 degrees  second reference(face 1 % 360 deg) face 1 on which face to sketch the next profile and change the angle on which angle we want the profile.

- Draw a rectangle on the reference plane 41*21mm

- Create a shaft cut for the servo motor shaft : sketch-> circle ->18 dia

ROBOTIC ARM ELBOW

- Draw to circles with dia of 42 and extrude to 5mm go to sketch select spine and connect the circles  extrude to 16mm

- Sketch a circle of 42mm and extrude cut-> fillets-> select fillets -> select the edges for fillet and select ok.

- Create screw holders for elbow and extrude cut

ROBOTIC ARM ELBOW 2

- Create a rectangle as shown in fig: 120mm and 38mm and extrude 15mm , create another rectangle for servo motor and extrude cut

- Create slot for micro servomotor on the other side with the following dimensions and extrude cut and create screw slots for fixing servo motor and apply fillets to the model

PP